- Exam questions

- How processor is implemented and what devices make up the processor? Describe the functions of these devices

- What effect does data word extension have on a computer? Why the first processor word was only 4 bits long. What is the main advantage of 64-bit word length over 32-bit word length?

- Which processors are called superscalar and which are VLIW processors? Briefly describe Super-scalability

- What measures increase CPU performance? List all.

- Explain the impact of downsizing (reduction) of the technological norm on processor performance.

- Explain the principles of operation of the conveyor and provide the benefits. How a Superconveyor is differ from a Conveyor

- Explain the essence of non-sequential command initiation and implementation conditions.

- Explain the nature of registry changes and their impact on CPU performance.

- Why processor needs a transition forecast in a conveyor processor?

- Explain the essence and benefits of data preloading.

- What variants of parallel calculations do you know? List and briefly describe

- What caused the use of multiple cores in the CPU? Explain the pros and cons

- What do we call a "single crystal system"? What can be loaded into a processor crystal?

- What are the obstacles to the smooth operation of the conveyor? What are the reasons for the obstacles? How can their influence on the operation of the conveyor be reduced?

- Describe the hierarchy of the memory system and the reasons why such hierarchy was created?

- What feature of a memory system is called locality? What ways do you know that locality manifests itself?

- What is semiconductor memory called static (SRAM)? Briefly describe the principle of operation?

- What kind of semiconductor memory is called dynamic DRAM? Briefly describe the principle of operation?

- What types of DDR memory do you know? Why is their bandwidth different?

- What are the main time parameters describing the performance of DDR memories? Comment that time parameters.

- What is SIMM? What is a DIMM?

- What is the difference between registered memory and non-buffered modules? Where is each of these types of modules used?

- What methods are used to ensure the correctness of information storage in memory? Shortly explain them.

- What is the Heming Code? What is the length of this code?

- Why computer use a DRAM controller? Who needs and how to use multi-channel memory?

- Describe the type of HMC memory.

- What kind of memory is called cache memory? Why do modern processors use multi-level caching?

- What questions are needed to answer to understand the cache working principle when using multiple memory levels?

- What are the main types of cache you know? Briefly describe them

- How is the cache used to select a data word?

- What line replacement strategies do you know? Describe known strategies

- Explain the idea of using track cache.

- Explain the nature of the information compatibility problem that arises in modern multi-core or multi-processor systems. Why processors needs special schemes to track the exchange of information in cahce (snooping hardware).

- Explain the idea of the MESI protocol?

- What are the reasons for not caching (hit-loss)? What effect does the length and type (associativity) of cache row have on the frequency of failures?