VILNIUS UNIVERSITY
ŠIAULIAI ACADEMY

BACHELOR PROGRAMME SOFTWARE ENGINEERING

Computer networks laboratory work No. 4

Student : Hanna Asiadouskaya

Lecturer: dr. Nerijus Ramanauskas

Šiauliai, 2024

- 192.168.0.131

- Value in upper layer protocol field is 1

- As in there 20 bytes in the IP header and the total length is 56 bytes, then in the payload would be 36 bytes.

- The “More fragments” flag is set to 0, that means that the datagram has not been fragmented.

- The header checksum, Time to Live and the Identification always changes.

- Stay Constant:

- Version because we are using IPv4 for all packets

- Length of the header because we are using IPv4

- Source IP since my computer’s IP address doesn’t change

- Destination IP as usc.edu’s IP address doesn’t change

- Differentiated services as there the same protocol every time

- Upper Layer Protocol since these are ICMP packets

Must stay Constant:

- Version

- Length of the header

- Source IP

- Destination IP

- Differentiated services

- Upper Layer Protocol

Must Change:

- Identification because IP packets must have different ids

- Time to live as it is increments with each packet

- Header checksum, as header also changes.

- The pattern in the identification field is that the field increases by one with each ICMP Echo request.

- Identification: 0xdb6f (56175); TTL: 1

- The Identification field changes from all of the replies because this field has to have a unique value. However, TTL field remains the same because the TTL for the first hop router is always the same, and equals 1.

- This packet has been fragmented across more than one IP datagram, but wireshark grouped them together

The fragment offset is 0, and the more fragments set flag is 1, so we know that this is the first fragment. The first datagram has a total length of 1500, including the header.

The fragment offset is 1480, and the more fragments set flag is 0, so we know that this is the last fragment.

- Total length, flags, fragment offset, and checksum are fields that changed.

- After switching to packet size of 3500 3 packets are created from the datagram

- The IP header fields that changed between all of the packets are fragment offset and checksum.